iDirect Technical Note

Subject:	Installing the iSCPC-Modem Images

Date:		August 13, 2003

# Introduction

This technical note describes the steps for installing the iDirect iSCPC-Modem Images and related software on a PC running Windows.  It also covers the steps needed to flash the iSCPC images on a NetModem, and configuring the NetModem for iSCPC operation.

# Detailed Information

#### Obtaining the iSCPC-Modem Images Software

The iSCPC-Modem Images software can be distributed in a number of ways.  Most likely, you will be given login access to the iDirect’s support website, http://tac.iDirect.net/.  If you have an FTP server that iDirect can log onto, then we can transfer the software to you by FTP.  Lastly, we can copy the software to a CD and ship the CD to you.

#### Components of the iSCPC-Modem Images Software

- The NMS NetManager program and support files

- The NMS Service Monitor program and support files

- The NetModem2 iSCPC-Modem image files

- The NetModem2Plus iSCPC-Modem image files

- The ISCPC-Modem Application

#### Installing the iSCPC-Modem Images and Software

The steps for installing the software depend upon whether you have received the software via the Internet or have a copy on CD.

- Installing software transferred via the Internet:

This software is usually distributed in a ZIP file.  Once you have the ZIP file you can extract the contents using one of the several Windows tools that recognize the .zip format such as Info-Zip (at http://www.info-zip.org/), WinZip (at http://www.winzip.com/), or PKZIP (at http://www.pkware.com/).

After you have gotten the compressed archive file iSCPC-<version>.zip from iDirect, copy it to a directory where you may uncompress the files.  (<version> is the NetModem version number in the form '3_0_12', '3_1_6', '3_2_0', etc.)

- Uncompress the files by using unzip, WinZip, PKZIP, or another utility that understands the .zip format.

The following directory structure will be created when the files are uncompressed:

NetModem-<version>/

NetManager/

ServiceMonitor/

Images/iSCPC_2plus/

Images/iSCPC_NM2/

- Create a shortcut to NetManager.exe (in the NetManager folder) and ServiceMonitor.exe (in the ServiceMonitor folder) on the desktop or Windows taskbar, if desired.

- Installing the software from the iDIRECT NetModem CD:

- Put the iDirect NetModem CD into the CD drive, and locate the file iSCPC-<version>.zip in the directory NetModem-<version> on the CD.  (<version> is the NetModem version number in the form '3_0_12', '3_1_6', '3_2_0', etc.)

- Uncompress the files in the .zip file to a folder on your hard drive by using unzip, WinZip, PKZIP, or another utility that understands the .zip format.

The following directory structure will be created when the files are uncompressed:

NetModem-<version>/

NetManager/

ServiceMonitor/

Images/iSCPC_2Plus/

Images/iSCPC_NM2/

- Create a shortcut to NetManager.exe (in the NetManager folder) and ServiceMonitor.exe (in the ServiceMonitor folder) on the desktop or Windows taskbar, if desired.

#### Flashing the iSCPC-Modem Images

The steps needed to flash the iSCPC images vary slightly depending on whether they are being flashed on a NetModem II, or a NetModem II+.  The only difference is the location of the images.  The NetModem II images are in a folder called ‘Images/iSCPC_NM2/’ and the NetModem II+ images are in a folder called ‘Images/iSCPC_2Plus/’.

  - Change the NetModem’s IP Address (if necessary)

    - Connect to the NetModem via the Serial Console

    - Type: laninfo <ip-address> <netmask> <gateway>

Ex: laninfo 192.168.0.1 255.255.255.0 0.0.0.0

    - Type “Reset”

  - Start NetManager.exe using the shortcut that was created earlier.

    - Enter the IP Address of the NetModem

    - Enter the password (default password is “iDirect”)

  - Download the correct image set (NetModem II or II+)

    - Click: “Configure” – “Download Modem Images”

    - Click: “Select”

    - Locate the correct image set for the platform (II or II+)

    - Click: “Open”

    - Select all the available checkboxes (all of them)

    - Click: “Download” and when done “Reset”

#### Configuring then NetModem for iSCPC Operation

This section documents the minimal number of steps needed in order to get two iSCPC NetModems to lock to each other over an I/F network.  The final configuration steps needed for an R/F network will be documented later.

You must already know the following things:

  - The correct L-Band frequencies.

  - The ‘bitrate’ and ‘fecblocks-per-frame’ for both the outroute and inroute.

NOTE:  Use the iSCPC Calculation Tool (Excel Spreadsheet) to

determine the transmit and receive bitrates (bitrate =

transmission/channel rate) as well as the transmit and

receive FEC Blocks.

- Configuring the NetModem for I/F operation:

- Connect to the NetModem via the Serial Console or Telnet

  - Type: su

  - Enter the correct password (default password is “iDirect”)

  - Type: install set defaults

  - Type: install set modem type peer

- Verify that the IP Address is correct, or change it by doing this:

  - Type: install set eth0 address 192.168.0.1

  - Type: install set eth0 netmask 255.255.255.0

  - Type: install set eth0 gateway 0.0.0.0

(use values that are appropriate to your network)

- Set the RX parameters (must match the Peer NetModems TX settings)

  - Type: install set rx frequency 1400000000

  - Type: install set rx acqrange 1000000

  - Type: install set rx bitrate 4560974

  - Type: install set timeplan in fecblocks 175 ( Use iSCPC Calculation Tool to determine FEC Blocks)

- Set the TX parameters (must match the Peer NetModems RX settings)

  - Type: install set tx frequency 1410000000

  - Type: install set tx bitrate 4560974

  - Type: install set tx power –25

  - Type: install set timeplan out fecblocks 175 ( Use iSCPC Calculation Tool to determine FEC Blocks)

- Save the settings and reset the NetModem

  - Type: install save and reset

Repeat these steps for the Peer NetModem.  Make sure the IP Address information is different (no network overlap).  Once the Peer NetModem has been completed, you could cross connect the TX/RX ports between then 2 remotes and verify that they lock to each other and that they can ping each other.

- Configuring Static Routes (if necessary):

- Connect to the “Hub” side NetModem via the Serial Console or Telnet

  - Type: su

  - Enter the correct password (default password is “iDirect”)

- Add a static route

  - Type: install add route <network> <netmask> <nexthop> <if>

Ex: install add route 192.168.1.0  255.255.255.0  0.0.0.0 sat0

- Enable RIP (if needed)

  - Type: install set rip enabled on

- Deleting static routes (if needed)

  - Type: install del route <network> <netmask>

Ex: install del route 192.168.1.0  255.255.255.0

The example values used above assume that the “Hub” side NetModem has been given the IP address 192.168.0.1/24, and that the “Remote” side NetModem has the IP address 192.168.1.1/24.  The static route(s) may also have to be configured on up-stream routers as well (if RIP is not enabled, or not supported).  The “Hub” side NetModem will typically also be configured with a default gateway (see step A.2.c above).

- Configuring the NetModem for R/F operation:

- Connect to the NetModem via the Serial Console or Telnet

  - Type: su

  - Enter the correct password (default password is “iDirect”)

- Verify the Out-Door-Unit settings

  - To turn on DC Power do this (only if needed):

    - Type: install set odu rx power on

-and/or-

    - Type: install set odu tx power on

  - To set the correct TX power do this:

    - Type: install set tx power <value>

The installation of the iDirect iSCPC NetModem is now complete.